������������������������������������������������������������������������������Ŀ

��Ŀ  ���Ŀ ��Ŀ � ���Ŀ ���Ŀ ����Ŀ ڿ  �

���� ���   ô � � ���   ���Ĵ   ô   ���Ĵ

����� ����� �� ��� ����� ��  �   ��   ��  �

����Ŀ ڿ  � ���Ŀ

ô   ���Ĵ ���

��   ��  � �����

���Ŀ ���Ŀ ����Ŀ ���Ŀ ڿ ��Ŀ � ���Ŀ

����� ���   ô � � ���Ĵ ô ô � � ���¿

�� �  ����� �� � � ��  � �� �� ��� �����

Magazine

This phile dated 03/12/1994

Volume 1, Issue 8

You can always find BTR on PsychoTron BBS

��������������������������������������������������������������������������������

Hello, welcome to but another edition of Beneath The Remains Magazine.

Volume 1 issue 8 will be on The Silver Box

The Silver Box:)

-=-=-=-=-=-=-=-=

How to turn your Touch-tone phone into a silver box.

1...* Unscrew your phone ( The Touch tone, desktop kind)

2...* Remove the mounted pad and take clear plastic cover from the bottom.

3...* Hold the pad with the numbers *, 0, and # facing you, and turn it upside

down so you can see the yellow PC board.  You should see 2 round black

doughnuts.

4...* Position the board so the solder points for the left doughnut face you.

5...* Count over 4 points from the left and attach a Green wire to that point.

6...* Between you and the doughnuts, there should be 2 long yellow capacitors.

to the right of these, and on the edge of the board there should be 3

gold contacts. We're going to use the one on the left.

7...* The contact originally is spot welded, so cut it open

8...* To the one nearest you, attach a red wire, to the other one a yellow wire

9...* Run the wires out of the phone, and solder the switch

10..* The orentation should be red to the center. the switch will now alternate

between normal and 1633Hz fourth column tones.

Look at this graph below for the actual freqs.

Dual-Tone Multi-Frequency  (Touch-Tone)   Frequencies

Key     Freq. #1  []    Freq. #2

[]

1        697     []     1209

2        697     []     1336

3        697     []     1477

A        697     []     1633

4        770     []     1209

5        770     []     1336

6        770     []     1477

B        770     []     1633

7        852     []     1209

8        852     []     1336

9        852     []     1477

C        852     []     1633

*        941     []     1209

0        941     []     1336

#        941     []     1477

D        941     []     1633

After your done with your Silver box, make sure it works, then call

NPA-555-1212 When it starts ringing hold down the 'D' key, if you get

a pissed off operator, then forget it, that NPA doesnt work.  Try

out of the way states like Nebraska.  Once you do it and it works

you will hear a pulsing tone. You can connect to a loop, if you

dial 6 and your friend dials 7.

-+\+-

A BTR production

All rights lost.

-rk